TECHNO-COMMERCIAL OFFER

BG "ECOX-ZLD" SYSTEM

CAPACITY: 100 KLD

| Quote Reference | BG/ECOX-ZLD/26-27/XXXX R0 |
| --- | --- |
| Quotation Date | 2026-04-20 |
| Submitted to | M/s. Client Name Ltd |
| Location | City |
| Prepared By | Engineer Name |
| Contact Details | 9154971801 / 9959477028 |
| E-mail | evs@bgengineeringind.com |

To,

M/s. Client Name Ltd

City

Kind Attn.: Mr. Name, Designation

Subject: Proposal for 100 KLD STRIPPER, MEE & ATFD System

Dear Sir,

This is with reference to discussions dated recent meetings, we are pleased to submit our offer for Design, Engineering, Manufacturing, Supply, Installation and Commissioning of 100 KLD STRIPPER, MEE and ATFD systems.

Please find our proposal for your kind perusal, we hope our offer is in line with your requirement.

Please feel free to contact us for in further information/clarification, if required.

For, B&G Engineering Industries

N S HARIBABU

Managing Partner

OFFER INDEX

| PART | CONTENT |
| --- | --- |
| PART I | EXECUTIVE SUMMARY |
| PART II | PROCESS DESCRIPTION |
| PART III | PROCESS FLOW DIAGRAM |
| PART IV | ESTIMATED PLANT ECONOMICS & OPEX |
| PART V | TECHNICAL DETAILS & UTILITIES |
| PART VI | SCOPE OF SUPPLY |
| PART VII | BATTERY LIMITS |
| PART VIII | SCOPE MATRIX |
| PART IX | BASIS OF COMMISSIONING / TAKE-OVER |
| PART X | PRICE AND COMMERCIAL TERMS & CONDITIONS |
| PART XI | GENERAL TERMS & CONDITIONS |

PART – I

EXECUTIVE SUMMARY

B&G Engineering Industries is an established Indian engineering company specializing in the design, manufacture, and supply of process equipment and turnkey solutions for the chemical, pharmaceutical, food, agro-processing, and wastewater treatment industries. B&G has built a strong reputation in critical separation and thermal systems such as evaporators, dryers, heat exchangers, distillation columns, solvent recovery system, and Zero Liquid Discharge (ZLD) plants. With in-house fabrication, testing, and quality control, B&G delivers customized, high-reliability solutions aligned with stringent industrial and environmental standards.

B&G's core strength lies in its process engineering capability optimizing energy efficiency, recovery of valuable by-products, and compliance with increasingly strict pollution norms. Our solutions are widely used in effluent concentration, solvent recovery, and product purification applications. Supported by an experienced engineering team and a growing domestic and export footprint, B&G Engineering Industries positions itself as a cost-effective and technically competent alternative to global OEMs, particularly for mid-scale and large industrial process plants.

PART – II

PROCESS DESCRIPTION

Stripper System

High COD stream effluent is fed to the MEE preheater where it is preheated and is fed to Stripper column. Heat required for stripper reboiler will be provided through steam. Solvent and Water mixed Vapour from top of the column is condensed in condenser(s).

Total Condensate is collected in reflux tank and Fraction of the condensate is fed back as reflux while remaining condensate is taken out as distillate and column bottom is pumped to Evaporation system for further process.

Multi-Effect Evaporator

The Feed from Stripper bottom is fed to MEE, where Evaporation will be concentrated in 4-effect evaporator plant. Feed will flow in forward feed manner in evaporators. All effects shall be of Shell & Tube Forced Circulation type.

Live dry saturated steam shall be fed to first effect shell side. The system will be subjected to vacuum and the Vapours generated in the first effect & vapour generated from ATFD shall be condensed on the shell side of second effect; vapours generated in the second effect shall be condensed on the shell side of third effect; Vapour generated in the third effect shall be condensed on the fourth effect; Vapours generated in the last effect shall be condensed on the shell side of surface condenser which is circulated with cooling water on tube side.

The total condensate shall be collected in condensate tank from surface condenser and will be treated further. Concentrated product after water evaporation shall be transferred to ATFD for further drying.

Agitated Thin Film Dryer (ATFD)

MEE Concentrate shall be stored in ATFD Feed Tank. ATFD Feed Tank shall be provided with low rpm agitator. Effluent from ATFD Feed Tank will be fed to the top of ATFD through ATFD Feed Pump or fed directly to ATFD through concentrate discharge pump.

Dry & saturated steam will be supplied in the ATFD Jacket which will heat the surface. ATFD is the vertical jacketed vessel fitted with rotating blades. Specially designed feed distribution system will form the thin film of Effluent along the inner wall of ATFD shell. Heat transfer from ATFD jacket to effluent film results in evaporation of film. Due to evaporation dry powder will form which will be scraped out by rotating blades.

The vapor evaporated from the ATFD will have significant amount of energy, that energy shall be recovered by utilizing the ATFD vapor in the evaporation system as heating medium. This energy recovery will save fresh steam consumption in evaporation system. This will also lead to lower the carbon foot print.

Continuous scraping of powder by rotating blades from ATFD shell will helps to create new heat transfer area for further evaporation. Dry product will be collected at the bottom of ATFD.

PART – III

PROCESS FLOW DIAGRAM

[Process Flow Diagram to be inserted by user]

Flow: Effluent Feed → Stripper Column → Multi-Effect Evaporator → ATFD → Dry Solids

PART – IV

ESTIMATED PLANT ECONOMICS & OPEX

BG ECOX-ZLD SYSTEM ADVANTAGE

| Method | Conventional Evaporation | BG ECOX-ZLD | Benefit |
| --- | --- | --- | --- |
| MEE Steam Consumption | 1155 Kg/h | 730 Kg/h | Steam reduction up to ~37% |
| Annual Steam Usage | 6930 tons/year | 4380 tons/year | 2550 tons/year (Savings) |
| Operating Cost | ₹1.39 Cr/year | ₹0.88 Cr/year | ~₹51 Lakhs/Year (Savings) |

Note: The above figures are based on 20 hrs/Day Operation, 300 Days/Year, and Steam cost of ₹2/Kg. Figures are tentative and indicative.

PART – V

TECHNICAL DETAILS & UTILITIES

Feed Parameters

| Parameter | UOM | Value |
| --- | --- | --- |
| Feed / Capacity | KLD | 100 |
| Feed pH | - | 6.5 - 7.5 |
| Total COD | PPM | 100,000 |
| Volatile Organic Solvents | PPM | 70,000 |
| Total Solids | % w/w | 7 |
| Suspended Solids | PPM | <500 |
| Feed Temperature | °C | 30 |
| Total Hardness | PPM | <500 |
| Silica | PPM | <20 |
| Free Chloride | PPM | Nil |
| Feed Nature | - | Non-Foaming |

System Technical Specifications

| Section | Parameter | Value |
| --- | --- | --- |
| Stripper System | Type | Tray Type Column |
|  | Feed Inlet (Kg/h) | 5000 |
|  | Top Distillate Out (Kg/h) | 490 (70% Solvents, 30% Water) |
|  | Stripper Bottom Out (Kg/h) | 4510 |
| Evaporator | Type | 4-Effect Multiple Effect Evaporator (Forced Circulation Type) |
|  | Feed Inlet (Kg/h) | 4510 |
|  | Feed Solids (%) | 7.77 |
|  | Water Evaporation (Kg/h) | 3650 |
|  | Concentrate Out (Kg/h) | 860 (40%) |
| ATFD | Type | Agitated Thin Film Dryer |
|  | Feed Inlet (Kg/h) | 860 |
|  | Feed Solids (%) | 40 |
|  | Water Evaporation (Kg/h) | 485 |
|  | ATFD Product Out (Kg/h) | 375 |
|  | Moisture in Product (%) | ~8-10 |

Utilities Specification

| Utility | Operating Parameter | Unit | Consumption |
| --- | --- | --- | --- |
| Steam — Stripper | 1.5 Bar-g, >96% dryness | Kg/h | 510 |
| Steam — MEE | 1.0 Bar-g, >96% dryness | Kg/h | 730 |
| Steam Economy for MEE |  | Kg/Kg | ~5.0 |
| Steam — ATFD | 7.0 Bar-g | Kg/h | 600 |
| Power Consumption | 415V, 50 Hz, 3 Phase | kWh | 85 |
| Power Installed (incl. standby) | 415V, 50 Hz, 3 Phase | kW | 140 |
| Cooling Water (closed loop) | In/Out: 32 / 38 Deg. C | m³/h | 190 |
| Seal Water Re-circulation | In/Out: 30 / 32 °C | m³/h | DDE |
| Compressed Air | 6 Bar-g | Nm³/h | 6-8 |
| CIP Solutions | 1.5% HNO3 Solution / 3.0% Caustic Solution 1 Bar, 75-85 Deg.C | m³ | DDE |

Performance Guarantee

System Performance shall be Guaranteed for below parameters with 10% tolerance:

Stripper: Top Distillate Outlet Flow as mentioned in above table

Evaporator: Water Evaporation Rate as mentioned in above table

ATFD: Water Evaporation rate as mentioned in above table

Steam consumption as mentioned in above table

Power consumption as mentioned in above table

System shall be cleaned as suggested by supplier during commissioning process.

Above mentioned energy consumption and performance are based on timely CIP of system.

Any change in feed parameters and utility shall impact the system performance.

Plant performance will depend on regular CIP and maintenance as per plant operation manual.

In case of variation in initial solids of feed on lower side, ATFD feed will be reduced, thereby vapor generation in ATFD will also be reduced. In such case ATFD vapor which is being used in evaporator as a heating medium shall be reduced and fresh steam consumption will increase.

PART – VI

SCOPE OF SUPPLY

I. Electro-Mechanical Items

Stripper System

| Equipment | Specification | Qty | B&G Scope | Buyer Scope |
| --- | --- | --- | --- | --- |
| Stripper Column (Tray Type) | Dia: 600 mm, Total Ht: 18-20 Mtr Column Shell: SS 316L Trays: SS 316L Body Flange: MS with SS Cladding | 1 No | ✓ | ✗ |
| Stripper Re-Boiler (FC Type) — Option 1 | HTA: 12 M² Shell: SS 304 Tube: 0.9 mm Thk, Titanium Gr2, Seamless Tube Sheet: Duplex 2205 Top & Bottom Bonnet: Duplex 2205 Body Flange: MS with Duplex Cladding | 1 No | ✓ | ✗ |
| Stripper Re-Boiler (FC Type) — Option 2 | Shell: SS 304 Tube: 38.1mm, 1.65 mm Thk, SS 316Ti Tube Sheet: SS 316L Top & Bottom Bonnet: SS 316L Body Flange: SS 316L | 1 No | ✓ | ✗ |
| Stripper Condenser | HTA: 25 M² Shell: SS 316L Tube: 25.4 mm, 1.6 mm, SS 316L Tube Sheet: SS 316L Top & Bottom Bonnet: SS 316L Body Flange: SS 316L | 1 No | ✓ | ✗ |
| Feed Pump | Flow: ~5 m³/h Type: Centrifugal, FLP, IE3 Single Mechanical Seal Contact Parts: SS 316, Base Frame: MS | 1W | ✓ | ✗ |
| Re-Boiler Re-Circulation Pump | Flow: ~100 m³/h Type: Centrifugal, FLP, IE3 Double Mechanical Seal Contact Parts: SS 316, Base Frame: MS | 1W+1SB | ✓ | ✗ |
| Re-Flux cum Condensate Pump | Flow: ~1 m³/h Type: Centrifugal, FLP, IE3 Single Mechanical Seal Contact Parts: SS 316, Base Frame: MS | 1W | ✓ | ✗ |
| Feed Balance Tank | Capacity: 1.5 KL Contact Parts: SS 316L Non-Contact Parts: SS 304/MS | 1 No | ✓ | ✗ |
| Reflux cum Condensate Tank | Capacity: 0.5 KL Contact Parts: SS 316L Non-Contact Parts: SS 304/MS | 1 No | ✓ | ✗ |
| Product Piping, Vapor Duct, Process Condensate Piping | Contact Parts: SS 316L/SS 304 Up to 50 NB: SCH 10 50-250 NB: SCH 5 Above 250 NB: Fabricated | 1 Lot | ✓ | ✗ |
| Cooling Water Piping | Contact Parts: MS | 1 Lot | ✗ | ✓ |
| Steam / IBR Piping with IBR Certification | MS – Class C | 1 Lot | ✗ | ✓ |

Note: W = Working, SB = Working Standby, SSB = Store Standby. Motors/Instruments per specification above.

Evaporation System (MEE)

| Equipment | Specification | Qty | B&G Scope | Buyer Scope |
| --- | --- | --- | --- | --- |
| Calandria (FC Type) — Option 1 | Total HTA: 325 M² Shell: SS 316L Tube: 0.9 mm, Titanium Gr2, Seamless Tube Sheet: Duplex 2205 Bonnet: Duplex 2205 | 4 Nos | ✓ | ✗ |
| Calandria (FC Type) — Option 2 | Total HTA: 295 M² Shell: SS 316L Tube: 38.1 mm, 1.6 mm Thk, SS 316Ti Tube Sheet: SS 316L Bonnet: SS 316L | 4 Nos | ✓ | ✗ |
| Pre-Heater | HTA: DDE Shell: SS 316L Tube: 25.4 mm, 1.6 mm Thk, SS 316Ti (Option 2) or 0.9mm Titanium (Option 1) | 4 Nos | ✓ | ✗ |
| Surface Condenser | HTA: 60 M² Shell: SS 316L Tubes: 25.4 mm, 1.6 mm, SS 316L Tube Sheet: SS 316L | 1 No | ✓ | ✗ |
| Vapor Liquid Separator | VLS-1: ~1 KL, VLS-2: ~2.5 KL VLS-3: ~3 KL, VLS-4: ~4 KL Contact Parts: SS 316L | 4 Nos | ✓ | ✗ |
| Recirculation Pumps | Type: Centrifugal, IE3, NFLP Double Mechanical Seal Contact Parts: SS 316 Base Frame: MS | 4W+2SSB | ✓ | ✗ |
| Concentrate Discharge Pump | Flow: 1 m³/h Type: Centrifugal, IE3, NFLP Double Mechanical Seal Contact Parts: SS 316 | 1W | ✓ | ✗ |
| Process Condensate Pump | Flow: 4 m³/h Type: Centrifugal, IE3, NFLP Double Mechanical Seal Contact Parts: SS 316 | 1W | ✓ | ✗ |
| Vacuum Pump | Model: VWS 150 Type: Water Ring Impeller: SS 316, Housing: MS | 1W+1SB | ✓ | ✗ |
| Process Condensate Tank | Capacity: 1.0 KL Contact Parts: SS 304 | 1 No | ✓ | ✗ |

Note: W = Working, SB = Working Standby, SSB = Store Standby. Motors/Instruments per specification above.

ATFD System

| Equipment | Specification | Qty | B&G Scope | Buyer Scope |
| --- | --- | --- | --- | --- |
| Agitated Thin Film Dryer (22 m²) | Steam Jacket: MS Inner Shell & Blades: SS 316L Rotor & Shaft: SS 316L Motor: Crompton, NFLP Gear Box: Bonfiglioli, Inline | 1 No | ✓ | ✗ |
| ATFD Condenser | HTA: 30 M² Shell: SS 316L Tube: 25.4 mm, 1.6 mm, SS 316L Tube Sheet: SS 316L | 1 No | ✓ | ✗ |
| Scrubber Column | Contact Parts: SS 316L Non-Contact parts: SS 304 | 1 Set | ✓ | ✗ |
| Feed Pump | Flow: 1 m³/h Type: Centrifugal/Screw, IE3, NFLP Single Mechanical Seal Contact Parts: SS 316 | 1W | ✓ | ✗ |
| Process Condensate Pump | Flow: 1 m³/h Type: Centrifugal, IE3, NFLP Single Mechanical Seal Contact Parts: SS 316 | 1W | ✓ | ✗ |
| Centrifugal Blower | 610 CFM Type: Centrifugal ID Fan Casing & Impeller: SS 304 | 1W | ✓ | ✗ |
| Feed Tank with Agitator | Capacity: 0.5 KL Contact Parts: SS 316L | 1 No | ✓ | ✗ |

Note: W = Working, SB = Working Standby, SSB = Store Standby. Motors/Instruments per specification above.

II. Instruments & Automation Items

| Item Description | Quantity | Scope |
| --- | --- | --- |
| Level Control Valve | 2 No | B&G |
| Magnetic Flow Meter | 4 No | B&G |
| Temperature Transmitter | 1 Lot | B&G |
| Pressure Transmitter | 3 No | B&G |
| Level Transmitter | 2 No | B&G |
| Vacuum Transmitter | 5 No | B&G |
| Rota-meter | 2 No | B&G |
| Pressure Gauges / Temperature Gauges / Vacuum Gauges | 1 Lot each | B&G |
| Control Panel: PLC with SCADA | 1 Set | B&G |
| MCC / Electrical Panel (NFLP) | 1 Set | B&G |
| Instrument Cables | 1 Lot | B&G |
| Power / Control / Data Cables between Motors / MCC / PLC | 1 Lot | Customer |
| Cable laying, Trays, Supports, Junction Boxes | 1 Lot | Customer |

III. Engineering & Executive Services

Basic Engineering

Detail Engineering

Software development

PFD / P&ID

GA Drawings and Layout

3D Drawing of Plant

Project Management

Installation

Pre-Commissioning

Commissioning & Training of system operators

Material Test Certificates

Hydro Test Certificates

Operating Manual

PART – VII

BATTERY LIMITS

Feed at the inlet of MEE Feed Balance Tank.

Steam at required pressure at the inlet of equipment nozzles along with IBR standard Piping, Bypass valve, isolation valves & steam traps.

Cooling water at 2.5 Bar-g at the inlet of condenser nozzles as required.

Seal water make-up at the Seal water tanks within battery limit.

Soft water for Cooling Tower make-up at inlet of CT basin.

Power at inlet of consumption point within battery limit.

Service water at one point of MEE area within battery limit.

Instrument air at one point of MEE area within battery limit.

Any chemicals for CIP solution preparation in customer scope.

Process Condensate handling from process condensate pumps.

Steam condensate from system outlet nozzles.

Dry product handling from ATFD bottom.

PART – VIII

SCOPE MATRIX

| S.No | Description of Work | B&G | CLIENT |
| --- | --- | --- | --- |
| 1 | Design, Engineering, Manufacturing, Supply, Installation and Commissioning as mentioned in scope of supply. | ✓ | ✗ |
| 2 | Unloading & Safe Storage of supplied material at site. | ✗ | ✓ |
| 3 | Supply of Pipes, fittings, valves within battery limits. | ✓ | ✗ |
| 4 | Fabrication & Installation of Pipes, fittings, valves within battery limits. | ✓ | ✗ |
| 5 | Insulation & Cladding of Stripper, MEE & ATFD system as required. | ✗ | ✓ |
| 6 | Supply of Pumps with Motors as mentioned in scope of supply. | ✓ | ✗ |
| 7 | Supply and Installation of MCC panel with Incoming/Outgoing Feeders, Busbars, Wiring, Starters, Drivers, Energy meters. | ✓ | ✗ |
| 8 | Supply & Installation of Instruments, Cables, PLC Panel with SCADA based Automation. | ✓ | ✗ |
| 9 | Electrical/Power Cables, Cable trays & Supports between MCC, PLC and Motors. | ✗ | ✓ |
| 10 | Cable trays, Cable laying between panel, instruments and valves. | ✗ | ✓ |
| 11 | Any civil related work (RCC tank, building, foundation, Platforms). | ✗ | ✓ |
| 12 | MS/Civil Structure with Platforms, Support Beams, Stair case, Handrails. | ✗ | ✓ |
| 13 | Plant shade roofing/sheeting, Louvers, Purlins, Foundation bolts. | ✗ | ✓ |
| 14 | Structure Load data, GA diagram, foundation drawings of plant. | ✓ | ✗ |
| 15 | Cooling tower with pumps & pipeline. | ✗ | ✓ |
| 16 | CIP system with CIP tank & pump, dosing tanks & pumps. | ✗ | ✓ |
| 17 | Seal water cooling system with Tank, PHE and pumps. | ✓ | ✗ |
| 18 | CIP/Effluent drain handling through pump, drain pit. | ✗ | ✓ |
| 19 | Any statutory approval, government and local authority approval. | ✗ | ✓ |
| 20 | HAZOP analysis, if required. | ✗ | ✓ |
| 21 | Plant lightning and any internal works. | ✗ | ✓ |
| 22 | Item/Service not mentioned in above scope of supply. | ✗ | ✓ |

PART – IX

BASIS OF COMMISSIONING / TAKE-OVER

To Carry out the Commissioning and Take-over of the Equipment/Plant, Buyer shall provide Operators/Supervisors, Sufficient quality and quantity of Materials, Utilities and necessary Consumables and continuous supply of Feed.

The Commissioning procedure by which the seller shall demonstrate that the equipment has met the take-over criteria shall be carried out by Buyer under the supervision of seller as per Operation Manuals provided by seller.

Seller shall demonstrate performance trial of Equipment/Plant maximum up to 48 hrs. This is Buyer's responsibility to provide continuous and uninterrupted supply of Feed, Utilities and Consumables.

When the commissioning of Equipment/Plant is completed or demonstrated, the Buyer shall take-over the equipment for the operation and maintenance thereof.

The Seller and the Buyer shall sign the takeover certificate thereafter Buyer shall be solely responsible for the safety, operation, service, maintenance of the equipment.

In the event of delay in completion of commissioning due to reasons not attributed to Seller from the period of 3 months from the date of mechanical completion, the Equipment/Plant shall be deemed to have been commissioned.

In case the performance guarantee doesn't achieve in the performance trial for the reason attributed to seller, allowing tolerance under performance guarantee, the seller shall be liable to pay liquidated damage subject to a maximum of 2.5% of Purchase order / Contract price.

PART – X

PRICE & COMMERCIAL TERMS AND CONDITIONS

Price Summary

Our total price for Design, Engineering, Manufacturing, Supply, Installation and Commissioning of Plant as per given Technical Specification and Scope of work (DAP, Hyderabad):

| S.N | Item / Equipment / Service | Option 1 — Titanium & Duplex 2205 | Option 2 — SS 316Ti & SS 316L |
| --- | --- | --- | --- |
| 1 | Design, Engineering & Supply of Stripper, MEE & ATFD System | Rs. 5.00 Cr | Rs. 4.85 Cr |
| 2 | Installation & Commissioning | Rs. 25 Lakhs | Rs. 25 Lakhs |
|  | TOTAL PRICE | Rs. 5.25 Cr | Rs. 5.10 Cr |

Prices are inclusive of Design, Engineering, Manufacturing, Supply, Installation, Commissioning, Transportation & P&F. Above prices are EXCLUDING Taxes and Duties, and Transit Insurance (charged extra at the time of dispatch).

Our above-mentioned price validity is 15 days from the date of this offer.

Terms of Payment

30% Advance of Basic order value.

65% of order value with applicable taxes & duties on Prorata basis against submission of proforma invoice after readiness of material but prior to dispatch.

5% remaining order value with applicable taxes & duties within 15 days after commissioning. In case of any delay in commissioning for reason not attributed to B&G Engineering, remaining amount shall be paid within 60 days from the date of last major supply.

Delivery Timeline (INCOTERMS 2020)

| Activity | Timeline |
| --- | --- |
| Supply (DAP, Hyderabad) – Option 1 | 6-7 Month from the date of PO and receipt of advance payment. |
| Supply (DAP, Hyderabad) – Option 2 | 3-4 Month from the date of PO and receipt of advance payment. |
| Installation | 1.5 Month (Subject to site readiness). |
| Commissioning | 15-20 Days. |

PART – XI

GENERAL TERMS & CONDITIONS

Buyer's Responsibilities

The Buyer shall supply all items and materials not specified as the responsibility of the Seller but which are necessary for the Seller to comply with its obligations under the contract. The Buyer shall provide access to the site suitable for the transport of the Equipment. The Seller shall not commence the Installation work unless the civil works are completed by Buyer and facilities like power, water and other essential utilities are made available at the Site by the Buyer. The Buyer shall be responsible for obtaining all licenses, permits, and approvals necessary.

Contract Price

The contract price quoted is Ex Works and exclusive of packing and forwarding charges, freight and insurance, taxes, duties and/or other levies or charges unless otherwise stated. All taxes, duties, levies and charges shall be billed at the rates in force at the time of dispatch.

Statutory Variations

Our prices are firm but subject to statutory variations. Any increase in Excise Duty, taxes etc. at the time of delivery shall be charged extra to your account.

Delivery

Unless otherwise stated, all supplies of the Equipment shall be Ex-works at Seller's works at Hyderabad as per Incoterms 2020. Time of delivery shall start from receipt of final Purchase Order and receipt of downpayment.

Transportation and Insurance

The Seller may arrange for transportation on 'freight to pay' or 'freight paid' basis. Buyer shall arrange insurance on the Equipment from the time the goods leave the Seller's works until commissioning at Buyer's factory site.

Mechanical Completion

As soon as the Equipment is substantially erected, the Buyer shall notify in writing to the Seller. Upon completion of demonstration, both parties shall sign the Mechanical Completion Certificate.

Mechanical Warranty

Each item of Equipment shall be free from defects in design, materials and workmanship for a period of 12 months from date of mechanical completion or 18 months from date of last major supply, whichever is earlier. This warranty is based on normal operation.

Force Majeure

Force Majeure shall include but not be restricted to Acts of God, action of government, strikes, floods, fires, earthquakes, explosions, accidents, epidemics, civil commotions, war, riots, and any factor beyond reasonable control. If Force Majeure affects performance exceeding 3 months, either party may terminate.

Terms of Payment

In case of delayed payment, Seller shall be entitled to interest at 15% or as agreed. In event of non-compliance with payment terms, Seller may enforce lien on Equipment already supplied and suspend performance.

Liability

Seller shall not be liable for loss of use, profits, revenue, contracts, or indirect or consequential losses. Maximum cumulative liability shall not exceed 5% of Contract Price.

Intellectual Property Rights

Seller shall provide back-up support to defend any lawsuits for alleged infringement of trademark or intellectual property rights with respect to this Offer.

Confidentiality

All data, information, designs, drawings, process know-how pertaining to the PLANT shall be kept confidential by Buyer. Buyer shall not disclose such information to third parties without written permission from Seller.

For B&G Engineering Industries

N S HARIBABU

Managing Partner